AEGIS Wristband

Firmware → Firebase Integration Guide

# 1. How data flows

The wristband sends vitals straight to the cloud over Wi-Fi — there is no intermediate phone or server. The ESP32 connects to Wi-Fi, authenticates to Firebase using a dedicated device account, and writes one document per reading directly into Cloud Firestore over HTTPS (TLS). The parent app reads those same documents in real time.

ESP32  --Wi-Fi/HTTPS-->  Firebase (Cloud Firestore)  -->  Parent App

If Wi-Fi drops, readings are encrypted and held in an on-device offline queue, then flushed automatically when the connection returns — no data is lost.

# 2. Configuration values (secrets.h)

Copy secrets.example.h to secrets.h and fill in the values below. secrets.h must never be committed to version control (it is already git-ignored).

| Field | Value |
| --- | --- |
| FIREBASE_PROJECT_ID | aegis-child-safety-app |
| FIREBASE_API_KEY | AIzaSyC-_Frb8dG9Oi4G6aKZEIb5teFbaZDHFVA |
| FIREBASE_HOST | firestore.googleapis.com (leave unchanged) |
| FIREBASE_USER_EMAIL | band-wajeehaa@aegis.device |
| FIREBASE_USER_PASSWORD | za8jwpxUV7hmTakS86kOKw |
| CHILD_ID | ecc086c4-215c-4661-b6c4-693cadb54d16 |
| USER_ID | (not used in paths — may be left blank) |
| WIFI_SSID | <the Wi-Fi network the band will use> |
| WIFI_PASSWORD | <that network's password> |

The device account and its permissions are already set up in the project; the email and password above are ready to use as-is. Only the two Wi-Fi values need to be supplied to match the deployment location.

# 3. Where the data is written

Vitals are written once per upload interval to:

children/{CHILD_ID}/vitals/{autoId}

Alerts (stress, geofence breach, low SpO2, etc.) are written to:

children/{CHILD_ID}/alerts/{autoId}

## 4. Vitals document fields

| Field | Type / meaning |
| --- | --- |
| hr | number — heart rate (BPM) |
| spo2 | number — blood oxygen (%) |
| gsr | number — skin conductance (µS) |
| temp | number — skin temperature (°C) |
| movement | number — acceleration magnitude |
| status | string — "NORMAL" ¦ "ELEVATED" ¦ "STRESS" |
| lat | number — GPS latitude (0 when no fix) |
| lng | number — GPS longitude (0 when no fix) |
| timestamp | string — ISO-8601 UTC, e.g. "2026-06-29T09:31:43Z" |
| battery | number — battery percentage (optional) |

## 5. Alert document fields

| Field | Type / meaning |
| --- | --- |
| type | string — "STRESS" ¦ "GEOFENCE" ¦ "SPO2" ¦ ... |
| timestamp | string — ISO-8601 UTC |
| vitals | map — { hr: number, spo2: number } |
| location | map — { lat: number, lng: number } |
| resolved | boolean — false on creation |

# 6. Setup checklist

1. Copy secrets.example.h to secrets.h and paste in the values from section 2.

2. Set WIFI_SSID and WIFI_PASSWORD to the network the band will run on.

3. Build and flash the firmware (the field/path changes only take effect after reflashing).

4. Power on the band within Wi-Fi range. After the first upload, documents appear under children/{CHILD_ID}/vitals and the app dashboard updates live.

# 7. Notes

No endpoint URL is required — the Firebase client library handles the REST call to Firestore internally.

All physiological and location data travels over HTTPS only; nothing sensitive is printed to the serial console in release builds.

Keep secrets.h out of git. It contains the device password.